Федеральное государственное автономное образовательное учреждение высшего профессионального образования «Национальный исследовательский университет «Высшая школа экономики»

Московский институт электроники и математики имени А.Н. Тихонова Национального исследовательского университета «Высшая школа экономики»

Департамент компьютерной инженерии

Курс «Высокоуровневое и имитационное моделирование цифровых систем»

ОТЧЕТ

о выполнении практической работы №3

тема работы: «Реализация нейронной сети MobileNet на ПЛИС»

Выполнили:

Бригада №6

Лукина А.С., БИВ174,

Целищев И.С., БИВ174

Принял:

асс. МИЭМ НИУ ВШЭ

Американов А.А.

Москва 2020 г.

Задание

Ознакомиться с материалами из руководств MobileNet start Guide.

Следуя инструкциям из MobileNet start Guide повторить пример подключения камеры и LCD экрана к ПЛИС DE0-Nano.

Произвести обучение нейронной сети.

Запустить проект в Quartus Prime, скомпилировать его и загрузить полученный .sof файл на ПЛИС.

Убедиться, что камера и монитор работают, нейронная сеть распознает изображения.

Выполнение работы

Обучение нейронной сети

Для обучения нейронной сети на компьютер были установлены Python 3.5, пакеты numpy, scipy и pyyaml и библиотеки TensorFlow и Keras.

В соответствии с мануалом MobileNet start Guide для инициализации и обучения нейронной сети был запущен скрипт r01_train_neural_net_and_prepare_initial_weights.py (предоставлен преподавателем). После обучения необходимо нормализовать веса и привести их к формату с фиксированной запятой. Для этого последовательно были выполнены скрипты r02_rescale_weights_to_use_fixed_points_representation.py (предоставлен преподавателем) и r03_find_optimal_bit_for_weights.py (предоставлен преподавателем).

Для предобработки изображений с камеры на ПЛИС с помощью скрипта r04_verilog_generator_grayscale_file.py (предоставлен преподавателем) был создан Verilog файл. В завершении был выполнен скрипт r05_verilog_generator_neural_net_structure.py (предоставлен преподавателем), генерирующий Verilog файл, описывающий структуру нейронной сети.

Отчет о компиляции проекта приведен на рис. 1.

Отчет о компиляции проекта

Подключение камеры и LCD экрана

На рис. 2 приведена схема подключения камеры и экрана к плате DE0-Nano. Результат подключения приведен на рис. 3, 4.

Схема подключения камеры и экрана к ПЛИС

Подключение камеры и экрана к ПЛИС (часть 1)

Подключение камеры и экрана к ПЛИС (часть 2)

Проверка работы нейронной сети

Для проверки работы нейронной сети к камере были поднесены изображения различных цифр (см. рис. 5, 6, 7, 8). Как видно на рис. 5, 6, 7, 8 нейронная сеть корректно распознает изображение цифр.

Распознавание цифры 9

Распознавание цифры 9 (другой шрифт)

Распознавание цифры 6

Распознавание цифры 3

Задание для самостоятельной работы

Задание

Выполнить миграцию на плату DE10-Standard.

Миграция проекта на плату

Для миграции на плату DE10-Standard в проект была добавлена новая версия проекта. (Для этого во вкладке Project была выбрана команда Revisions. После чего в открывшимся диалоговом окне была создана новая версия проекта. (см. рис. 9))

Версии проекта для разных плат

В Листинге 1 приведено содержание файла cam_proj_migration.qsf для новой версии проекта.

set_global_assignment -name FAMILY "Cyclone V"

set_global_assignment -name DEVICE 5CSXFC6D6F31C6

set_global_assignment -name TOP_LEVEL_ENTITY cam_proj_top

set_global_assignment -name ORIGINAL_QUARTUS_VERSION 17.1.0

set_global_assignment -name PROJECT_CREATION_TIME_DATE "16:03:09  NOVEMBER 28, 2020"

set_global_assignment -name LAST_QUARTUS_VERSION "17.1.0 Lite Edition"

set_global_assignment -name PROJECT_OUTPUT_DIRECTORY output_files

set_global_assignment -name MIN_CORE_JUNCTION_TEMP 0

set_global_assignment -name MAX_CORE_JUNCTION_TEMP 85

set_global_assignment -name ERROR_CHECK_FREQUENCY_DIVISOR 2

set_global_assignment -name POWER_PRESET_COOLING_SOLUTION "23 MM HEAT SINK WITH 200 LFPM AIRFLOW"

set_global_assignment -name POWER_BOARD_THERMAL_MODEL "NONE (CONSERVATIVE)"

set_global_assignment -name VERILOG_INPUT_VERSION SYSTEMVERILOG_2005

set_global_assignment -name VERILOG_SHOW_LMF_MAPPING_MESSAGES OFF

set_global_assignment -name STRATIX_DEVICE_IO_STANDARD "3.3-V LVTTL"

set_global_assignment -name USE_CONFIGURATION_DEVICE ON

set_global_assignment -name RESERVE_ALL_UNUSED_PINS "AS INPUT TRI-STATED"

set_global_assignment -name RESERVE_ALL_UNUSED_PINS_NO_OUTPUT_GND "AS INPUT TRI-STATED"

set_global_assignment -name ENABLE_SIGNALTAP OFF

set_global_assignment -name USE_SIGNALTAP_FILE output_files/stp1.stp

set_global_assignment -name VERILOG_MACRO "nornal=1"

set_location_assignment PIN_AF14 -to clk50

set_location_assignment PIN_AH12 -to sdram_clk

set_location_assignment PIN_AE9 -to tft_cs

set_location_assignment PIN_AF10 -to tft_dc

set_location_assignment PIN_AE7 -to tft_reset

set_location_assignment PIN_AF8 -to tft_sck

set_location_assignment PIN_AF9 -to tft_sdi

set_location_assignment PIN_AF5 -to tft_sdo

set_location_assignment PIN_AH13 -to sd_addr[11]

set_location_assignment PIN_AG12 -to sd_addr[10]

set_location_assignment PIN_AG13 -to sd_addr[9]

set_location_assignment PIN_AH15 -to sd_addr[8]

set_location_assignment PIN_AD14 -to sd_addr[6]

set_location_assignment PIN_AF15 -to sd_addr[7]

set_location_assignment PIN_AC14 -to sd_addr[5]

set_location_assignment PIN_AB15 -to sd_addr[4]

set_location_assignment PIN_AE14 -to sd_addr[3]

set_location_assignment PIN_AG15 -to sd_addr[2]

set_location_assignment PIN_AH14 -to sd_addr[1]

set_location_assignment PIN_AK14 -to sd_addr[0]

set_location_assignment PIN_AJ5 -to sd_data[15]

set_location_assignment PIN_AJ6 -to sd_data[14]

set_location_assignment PIN_AH7 -to sd_data[13]

set_location_assignment PIN_AH8 -to sd_data[12]

set_location_assignment PIN_AH9 -to sd_data[11]

set_location_assignment PIN_AJ9 -to sd_data[10]

set_location_assignment PIN_AJ10 -to sd_data[9]

set_location_assignment PIN_AH10 -to sd_data[8]

set_location_assignment PIN_AJ11 -to sd_data[7]

set_location_assignment PIN_AK11 -to sd_data[6]

set_location_assignment PIN_AG10 -to sd_data[5]

set_location_assignment PIN_AK9 -to sd_data[4]

set_location_assignment PIN_AK8 -to sd_data[3]

set_location_assignment PIN_AK7 -to sd_data[2]

set_location_assignment PIN_AJ7 -to sd_data[1]

set_location_assignment PIN_AK6 -to sd_data[0]

set_location_assignment PIN_AJ12 -to ba[1]

set_location_assignment PIN_AF13 -to ba[0]

set_location_assignment PIN_AK12 -to dqm[1]

set_location_assignment PIN_AB13 -to dqm[0]

set_location_assignment PIN_AE13 -to ras_n

set_location_assignment PIN_AA13 -to we_n

set_location_assignment PIN_AF11 -to cas_n

set_location_assignment PIN_AK13 -to Cke

set_location_assignment PIN_AG11 -to cs_n

set_location_assignment PIN_AA12 -to data_cam[7]

set_location_assignment PIN_AB12 -to data_cam[6]

set_location_assignment PIN_AC12 -to data_cam[5]

set_location_assignment PIN_AC9 -to data_cam[4]

set_location_assignment PIN_AD10 -to data_cam[3]

set_location_assignment PIN_AD11 -to data_cam[2]

set_location_assignment PIN_AD12 -to data_cam[1]

set_location_assignment PIN_AD9 -to data_cam[0]

set_location_assignment PIN_AG2 -to HREF_cam

set_location_assignment PIN_AG6 -to PCLK_cam

set_location_assignment PIN_AG1 -to VSYNC_cam

set_location_assignment PIN_AG7 -to XCLK_cam

set_location_assignment PIN_AE12 -to on_off_cam

set_location_assignment PIN_AH5 -to res_cam

set_location_assignment PIN_Y16 -to sioc

set_location_assignment PIN_AK2 -to siod

set_location_assignment PIN_AJ4 -to rst

set_global_assignment -name EDA_SIMULATION_TOOL "ModelSim (Verilog)"

set_global_assignment -name EDA_TIME_SCALE "1 ps" -section_id eda_simulation

set_global_assignment -name EDA_OUTPUT_DATA_FORMAT "VERILOG HDL" -section_id eda_simulation

set_location_assignment PIN_AF24 -to LED[7]

set_location_assignment PIN_AE24 -to LED[6]

set_location_assignment PIN_AF25 -to LED[5]

set_location_assignment PIN_AG25 -to LED[4]

set_location_assignment PIN_AD24 -to LED[3]

set_location_assignment PIN_AC23 -to LED[2]

set_location_assignment PIN_AB23 -to LED[1]

set_location_assignment PIN_AA24 -to LED[0]

set_location_assignment PIN_AK3 -to b[4]

set_location_assignment PIN_AD7 -to g[5]

set_location_assignment PIN_AE11 -to r[4]

set_location_assignment PIN_AK4 -to start_gray_kn

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to HREF_cam

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to PCLK_cam

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to VSYNC_cam

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to XCLK_cam

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to data_cam[7]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to data_cam[6]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to data_cam[5]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to data_cam[4]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to data_cam[3]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to data_cam[2]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to data_cam[1]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to data_cam[0]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to on_off_cam

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to res_cam

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sioc

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to siod

set_instance_assignment -name WEAK_PULL_UP_RESISTOR ON -to sioc

set_instance_assignment -name WEAK_PULL_UP_RESISTOR ON -to siod

set_global_assignment -name PARTITION_NETLIST_TYPE SOURCE -section_id Top

set_global_assignment -name PARTITION_FITTER_PRESERVATION_LEVEL PLACEMENT_AND_ROUTING -section_id Top

set_global_assignment -name PARTITION_COLOR 16764057 -section_id Top

set_global_assignment -name VERILOG_FILE ../code/synt/sdram_controller.v

set_global_assignment -name QIP_FILE pll.qip

set_global_assignment -name SYSTEMVERILOG_FILE ../code/lcd/tft_ili9341_spi.sv

set_global_assignment -name SYSTEMVERILOG_FILE ../code/lcd/tft_ili9341.sv

set_global_assignment -name SYSTEMVERILOG_FILE ../code/lcd/hellosoc_top.sv

set_global_assignment -name VERILOG_FILE ../code/synt/cam_config/SCCB_interface.v

set_global_assignment -name VERILOG_FILE ../code/synt/cam_config/OV7670_config_rom.v

set_global_assignment -name VERILOG_FILE ../code/synt/cam_config/OV7670_config.v

set_global_assignment -name VERILOG_FILE ../code/synt/cam_config/camera_read.v

set_global_assignment -name VERILOG_FILE ../code/synt/cam_config/camera_configure.v

set_global_assignment -name SDC_FILE cam_proj.out.sdc

set_global_assignment -name VERILOG_FILE ../code/synt/cam_wrp.v

set_global_assignment -name VERILOG_FILE ../top/cam_proj_top.v

set_global_assignment -name SIGNALTAP_FILE output_files/stp1.stp

set_global_assignment -name QIP_FILE ../code/synt/fifo_1024x16.qip

set_global_assignment -name CDF_FILE output_files/programmer.cdf

set_global_assignment -name QIP_FILE pll_for_disp.qip

set_global_assignment -name QIP_FILE fifo_big.qip

set_global_assignment -name VERILOG_FILE ../code/gray_28x28/grayscale.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/TOP.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/result.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/RAMtoMEM.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/RAM.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/maxpooling.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/dense.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/database.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/conv_TOP.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/conv.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/border.v

set_global_assignment -name VERILOG_FILE ../code/neuroset/addressRAM.v

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to Cke

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to LED[7]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to LED[6]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to LED[5]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to LED[4]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to LED[3]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to LED[2]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to LED[1]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to LED[0]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to b[4]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to ba[1]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to ba[0]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to cas_n

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to clk50

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to cs_n

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to dqm[1]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to dqm[0]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to g[5]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to r[4]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to ras_n

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to rst

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[11]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[10]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[9]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[8]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[7]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[6]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[5]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[4]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[3]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[2]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[1]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_addr[0]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[15]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[14]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[13]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[12]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[11]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[10]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[9]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[8]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[7]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[6]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[5]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[4]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[3]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[2]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[1]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sd_data[0]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to sdram_clk

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to start_gray_kn

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to tft_cs

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to tft_dc

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to tft_reset

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to tft_sck

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to tft_sdi

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to tft_sdo

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to we_n

set_instance_assignment -name PARTITION_HIERARCHY root_partition -to | -section_id Top

cam_proj_migration.qsf

На рис. 10 представлен результат компиляции новой версии проекта, на рис. 11 – отчет о сравнении двух версий.

Отчет о компиляции новой версии проекта

Сравнение двух версий проекта

Выводы

В данной лабораторной работе мы продолжили изучение сети MobileNet. Мы узнали, как реализовать нейронную сеть MobileNet на ПЛИС. В данной лабораторной работе мы подключили к плате камеру и LCD экран. Мы узнали, как производить обработку изображения, поступающего с камеры, на плате. Также мы научились выводить изображение на LCD экран. Для проверки работы сети к подключенной камере были поднесены изображения различных цифр, после чего на экране был произведен вывод распознанного значения. В завершении лабораторной работы нами была произведена миграция проекта на другую плату.